# AI-Assisted Aircraft Safety Prototype

Python AI + ESP32 Communication Code
Educational/tabletop research prototype only. Not for direct control of a real aircraft.

# 1. Project Architecture

Simulated Aircraft Sensors → Python AI Monitoring → USB Serial → ESP32 → Status Display / Safe Prototype Interface

# 2. Python AI Program

Filename: ai/aircraft_ai.py

"""
AI-Assisted Aircraft Safety Monitoring
--------------------------------------

Educational/tabletop simulation only.

The program:
1. Generates simulated aircraft sensor data.
2. Detects abnormal conditions.
3. Calculates an emergency status.
4. Sends STATUS information to an ESP32.

It does NOT directly command an aircraft or real
aircraft separation mechanism.
"""

import random
import time
import serial
from dataclasses import dataclass

SERIAL_PORT = "COM3"
BAUD_RATE = 115200


@dataclass
class AircraftData:
    temperature: float
    vibration: float
    cabin_pressure: float
    structural_strain: float
    fuel_percent: float
    engine_health: float
    altitude: float
    speed: float


def generate_sensor_data(emergency=False):

    if emergency:
        return AircraftData(
            temperature=random.uniform(80, 100),
            vibration=random.uniform(7, 10),
            cabin_pressure=random.uniform(55, 75),
            structural_strain=random.uniform(75, 100),
            fuel_percent=random.uniform(10, 50),
            engine_health=random.uniform(20, 50),
            altitude=random.uniform(3000, 10000),
            speed=random.uniform(150, 300)
        )

    return AircraftData(
        temperature=random.uniform(20, 45),
        vibration=random.uniform(0, 4),
        cabin_pressure=random.uniform(95, 102),
        structural_strain=random.uniform(10, 40),
        fuel_percent=random.uniform(40, 100),
        engine_health=random.uniform(80, 100),
        altitude=random.uniform(5000, 12000),
        speed=random.uniform(200, 450)
    )


def analyze_aircraft(data):

    warnings = []

    if data.temperature > 75:
        warnings.append("HIGH_TEMPERATURE")

    if data.vibration > 6:
        warnings.append("HIGH_VIBRATION")

    if data.cabin_pressure < 80:
        warnings.append("PRESSURE_ANOMALY")

    if data.structural_strain > 70:
        warnings.append("STRUCTURAL_ANOMALY")

    if data.fuel_percent < 15:
        warnings.append("LOW_FUEL")

    if data.engine_health < 50:
        warnings.append("ENGINE_DEGRADED")

    if len(warnings) >= 3:
        status = "CRITICAL"
    elif len(warnings) >= 1:
        status = "WARNING"
    else:
        status = "NORMAL"

    confidence = min(99, 70 + len(warnings) * 8)

    return status, confidence, warnings


def print_aircraft_status(data, status, confidence, warnings):

    print("\n===================================")
    print("      AIRCRAFT AI MONITOR")
    print("===================================")

    print(f"Temperature       : {data.temperature:.1f} °C")
    print(f"Vibration         : {data.vibration:.2f}")
    print(f"Cabin Pressure    : {data.cabin_pressure:.1f}")
    print(f"Structural Strain : {data.structural_strain:.1f}")
    print(f"Fuel              : {data.fuel_percent:.1f}%")
    print(f"Engine Health     : {data.engine_health:.1f}%")
    print(f"Altitude          : {data.altitude:.0f} ft")
    print(f"Speed             : {data.speed:.0f}")

    print("-----------------------------------")
    print(f"STATUS            : {status}")
    print(f"CONFIDENCE        : {confidence}%")

    if warnings:
        print("\nDetected anomalies:")
        for warning in warnings:
            print(" -", warning)
    else:
        print("No abnormal conditions detected.")


def connect_esp32():

    try:
        esp32 = serial.Serial(
            SERIAL_PORT,
            BAUD_RATE,
            timeout=1
        )

        time.sleep(2)
        print("\nESP32 connected.")

        return esp32

    except serial.SerialException:

        print("\nESP32 not connected.")
        print("Running AI simulation without ESP32.")

        return None


def send_status(esp32, status):

    if esp32 is None:
        return

    message = f"STATUS:{status}\n"

    esp32.write(message.encode("utf-8"))

    print(f"Sent to ESP32: {message.strip()}")


def main():

    print("AI Aircraft Safety Prototype")
    print("--------------------------------")

    esp32 = connect_esp32()

    try:

        while True:

            emergency = random.randint(1, 8) == 1

            data = generate_sensor_data(
                emergency=emergency
            )

            status, confidence, warnings = analyze_aircraft(
                data
            )

            print_aircraft_status(
                data,
                status,
                confidence,
                warnings
            )

            send_status(
                esp32,
                status
            )

            time.sleep(3)

    except KeyboardInterrupt:

        print("\nSimulation stopped.")

    finally:

        if esp32 is not None:
            esp32.close()


if __name__ == "__main__":
    main()

# 3. ESP32 Firmware

Filename: firmware/esp32_controller/esp32_controller.ino

/*
 * AI Aircraft Safety Prototype
 *
 * Educational/tabletop prototype only.
 *
 * Python sends:
 * STATUS:NORMAL
 * STATUS:WARNING
 * STATUS:CRITICAL
 *
 * The ESP32 displays the status.
 *
 * Any physical prototype actuator should remain
 * behind an independent manual safety interlock.
 */

String receivedData = "";

const int GREEN_LED = 25;
const int YELLOW_LED = 26;
const int RED_LED = 27;

void setup() {

  Serial.begin(115200);

  pinMode(GREEN_LED, OUTPUT);
  pinMode(YELLOW_LED, OUTPUT);
  pinMode(RED_LED, OUTPUT);

  setNormal();

  Serial.println("ESP32 Aircraft Safety Monitor");
  Serial.println("Waiting for AI data...");
}

void loop() {

  if (Serial.available()) {

    receivedData = Serial.readStringUntil('\n');
    receivedData.trim();

    Serial.print("Received: ");
    Serial.println(receivedData);

    if (receivedData == "STATUS:NORMAL") {
      setNormal();
    }
    else if (receivedData == "STATUS:WARNING") {
      setWarning();
    }
    else if (receivedData == "STATUS:CRITICAL") {
      setCritical();
    }
    else {
      Serial.println("Unknown command.");
    }
  }
}

void setNormal() {

  digitalWrite(GREEN_LED, HIGH);
  digitalWrite(YELLOW_LED, LOW);
  digitalWrite(RED_LED, LOW);

  Serial.println("SYSTEM STATUS: NORMAL");
}

void setWarning() {

  digitalWrite(GREEN_LED, LOW);
  digitalWrite(YELLOW_LED, HIGH);
  digitalWrite(RED_LED, LOW);

  Serial.println("SYSTEM STATUS: WARNING");
}

void setCritical() {

  digitalWrite(GREEN_LED, LOW);
  digitalWrite(YELLOW_LED, LOW);
  digitalWrite(RED_LED, HIGH);

  Serial.println("SYSTEM STATUS: CRITICAL");
}

# 4. Python Requirements

requirements.txt

numpy
pyserial
scikit-learn

# 5. How to Run

Install Python 3.

Install dependencies with: pip install -r requirements.txt

Open aircraft_ai.py and change SERIAL_PORT from COM3 to the port used by your ESP32.

Upload the ESP32 firmware using Arduino IDE.

Run: python aircraft_ai.py

The Python program generates simulated aircraft sensor data and sends NORMAL, WARNING, or CRITICAL status to the ESP32.

# 6. Safety Note

This code is for an educational/tabletop research prototype. It must not be connected to or used to control a real aircraft, real aircraft separation system, or other safety-critical aircraft hardware. The ESP32 example only displays AI status; any physical prototype actuator should remain behind an independent manual safety interlock.